# XRComm FCS Wideband Receive Platform

Application Development Guide

XRComm Inc.

June 2026

# Introduction

This document provides detailed guidelines for developers writing, compiling, and running custom applications that interface with the XRComm FCS Wideband Platform.

The platform provides a header-only, zero-copy C API (xrcomm_ip_client.h) for POSIX applications that consume I/Q sample batches, and supports DPDK secondary process attachment for full-packet processing applications. IQ applications do not link against DPDK or SPDK; those libraries are used by the platform services. Reference documentation is available from DPDK (https://core.dpdk.org/doc/) and SPDK (https://spdk.io/doc/).

# Compiling and Running the Hello World Examples

The platform includes two example consumer applications in the XRComm_wideband_hello_world/ folder that demonstrate how to subscribe to and consume platform data streams.

| Example Binary | Delivery Mode | Enable With | Privileges |
| --- | --- | --- | --- |
| hello_world | IQ samples (zero-copy shared memory) | set-ip on | None |
| hello_world_fullpkt | Full packets (DPDK secondary) | set-dsp on | sudo |

## Compile the Examples

From the XRComm_FCS_platform root directory:

cd XRComm_wideband_hello_world
mkdir -p build && cd build
cmake ..
make
cd ../..

Compiled binaries are placed in XRComm_wideband_hello_world/bin/.

## Run the IQ Example

The IQ example connects to the Platform Driver via zero-copy shared memory. It prints a rising count of received batches and calculated power levels (dBFS).

Terminal 1 (FlexCompute Server):

./XRComm_wideband_hello_world/bin/hello_world

Terminal 2 (Client Machine or FlexCompute Server):

python3 xrcomm_grpc_client.py --host <server-ip>:50051 set-ip on

## Run the Full-Packet Example

The full-packet example attaches to the Platform Driver’s memory pool as a DPDK secondary process to consume raw Ethernet frames. It requires root privileges.

Terminal 1 (FlexCompute Server):

sudo ./XRComm_wideband_hello_world/bin/hello_world_fullpkt

Terminal 2 (Client Machine or FlexCompute Server):

python3 xrcomm_grpc_client.py --host <server-ip>:50051 set-dsp on

## Verify the Application Is Connected

Query the registry from the Client Machine:

python3 xrcomm_grpc_client.py --host <server-ip>:50051 list-ips
# Output: slot=0 hello_world mode=IQ active

python3 xrcomm_grpc_client.py --host <server-ip>:50051 get-mode-stats
# Output: per-application received / processed / dropped counts

# Application API Reference

Include <xrcomm/xrcomm_ip_client.h> in your C/C++ applications. The header uses standard POSIX shared memory (shm_open, mmap) and does not require linking against DPDK libraries.

## Quick API Use Case: Consume Wideband IQ Samples

The usual customer application registers one IQ consumer, waits for the shared-memory ring, polls available batches, processes the interleaved int16 IQ samples, and releases each batch:

#include <stdio.h>
#include <xrcomm/xrcomm_ip_client.h>

int main(void) {
    xrcomm_ip_client_t client;
    if (xrcomm_ip_client_init(&client, "wideband_iq_app") != 0) {
        return 1;
    }
    if (xrcomm_ip_client_wait_ready(&client, 5000) != 0) {
        xrcomm_ip_client_shutdown(&client);
        return 1;
    }

    while (xrcomm_ip_client_running(&client)) {
        const xrcomm_iq_slot_t *slot = xrcomm_ip_client_poll(&client);
        if (!slot) {
            continue;
        }

        printf("samples=%u dropped=%lu\n",
               slot->valid_sample_count,
               xrcomm_ip_client_get_dropped(&client));
        xrcomm_ip_client_report_spectrum(&client, 1, 0.0f);
        xrcomm_ip_client_consume(&client);
    }

    xrcomm_ip_client_shutdown(&client);
    return 0;
}

Enable IQ delivery from the gRPC client with set-ip on before expecting batches.

## Lifecycle and Connection Functions

### xrcomm_ip_client_init

int xrcomm_ip_client_init(xrcomm_ip_client_t *client, const char *name);

Registers the application with the platform under the specified name and maps the primary control region. Returns 0 on success.

### xrcomm_ip_client_init_port

int xrcomm_ip_client_init_port(xrcomm_ip_client_t *client, const char *name, 
                               uint32_t hsp_port, uint32_t channel);

Registers the application and requests a specific NIC port (0 or 1) and channel (0..3, or 255 for all channels combined).

### xrcomm_ip_client_wait_ready

int xrcomm_ip_client_wait_ready(xrcomm_ip_client_t *client, int timeout_ms);

Blocks the caller until the Platform Driver has created and opened the shared memory ring buffer for this application.

### xrcomm_ip_client_running

bool xrcomm_ip_client_running(xrcomm_ip_client_t *client);

Returns true as long as the platform’s main receive loop is active. Use this function as the loop guard for sample ingestion.

### xrcomm_ip_client_shutdown

void xrcomm_ip_client_shutdown(xrcomm_ip_client_t *client);

Deregisters the application from the platform and unmaps/unlinks all shared memory regions.

## Data Consumer Functions

### xrcomm_ip_client_poll

const xrcomm_iq_slot_t* xrcomm_ip_client_poll(xrcomm_ip_client_t *client);

Polls the application ring buffer for a new batch of IQ samples. Returns a read-only pointer to the sample slot if data is available, or NULL if the ring is empty.

### xrcomm_ip_client_consume

void xrcomm_ip_client_consume(xrcomm_ip_client_t *client);

Releases the last slot returned by poll(), advancing the read pointer and returning the slot to the Platform Driver’s write pool. Must be called exactly once after a successful poll.

## Diagnostics and Telemetry

### xrcomm_ip_client_get_dropped

uint64_t xrcomm_ip_client_get_dropped(xrcomm_ip_client_t *client);

Returns the number of sample batches dropped by the Platform Driver since connection start due to the application’s ring buffer being full.

### xrcomm_ip_client_report_spectrum

void xrcomm_ip_client_report_spectrum(xrcomm_ip_client_t *client, 
                                      uint64_t processed, float power_dbfs);

Reports the application’s processing metrics (total batches consumed and current signal power) back to the Platform Driver. These values are displayed in the gRPC telemetry and statistics.