# XRComm FCS 8T8R Platform

Application Development Guide

XRComm Inc.

June 2026

# Introduction

This document provides detailed guidelines for developers writing, compiling, and running custom applications that interface with the XRComm FCS 8T8R Platform.

The platform provides a header-only, zero-copy C API (xrcomm_ip_client.h) for POSIX applications that consume I/Q sample batches. Customer applications do not include DPDK or SPDK headers for IQ delivery; those libraries are used by the platform services. Reference documentation is available from DPDK (https://core.dpdk.org/doc/) and SPDK (https://spdk.io/doc/).

# Compiling and Running the Hello World Examples

The platform includes an example consumer application in the XRComm_8T8R_hello_world/ folder that demonstrates how to subscribe to and consume platform data streams.

| Example Binary | Delivery Mode | Enable With | Privileges |
| --- | --- | --- | --- |
| hello_world | IQ samples (zero-copy shared memory) | set-ip on | None |

## Build the Hello-World Application

Navigate to the hello world folder:

cd XRComm_8T8R_hello_world/

Compile using CMake:

mkdir -p build && cd build
cmake ..
make

Or compile directly with GCC:

gcc -O3 -mavx2 -pthread -Wall -o hello_world hello_world.c \
    -I../XRComm_FCS_8T8R_platform/XRComm_platform_drivers/include \
    -lrt -lpthread -lm

Compiled binaries are placed in bin/ (if compiled with CMake) or the folder root (if compiled with GCC).

## Run the Hello-World Application

The platform driver must be running and IQ mode must be enabled before starting the application:

Terminal 1 (FlexServer): Ensure xrcomm_server is running, then enable IQ mode:

python3 XRComm_platform_gRPC/client/xrcomm_grpc_client.py set-ip on

Terminal 2 (FlexServer): Run the application:

./hello_world

The example prints a summary of batches, samples, signal power, and drops for all 8 channels:

[IP:hello_world_ch0] Registered slot 0 (pid 12345, port 0, ch 0)
...
[HW] All channels ready — starting receive loop
[HW] Channel summary (port 0):
  ch0: batches=15234  samples=487488  power=-18.4dBFS  mean=-18.6dBFS  dropped=0
  ch1: batches=15234  samples=487488  power=-19.1dBFS  mean=-19.2dBFS  dropped=0
  ch2: batches=15234  samples=487488  power=-18.9dBFS  mean=-19.0dBFS  dropped=0
  ch3: batches=15234  samples=487488  power=-18.7dBFS  mean=-18.8dBFS  dropped=0
[HW] Channel summary (port 1):
  ch4: batches=15234  samples=487488  power=-19.3dBFS  mean=-19.4dBFS  dropped=0
  ch5: batches=15234  samples=487488  power=-18.8dBFS  mean=-18.9dBFS  dropped=0
  ch6: batches=15234  samples=487488  power=-19.0dBFS  mean=-19.1dBFS  dropped=0
  ch7: batches=15234  samples=487488  power=-18.6dBFS  mean=-18.7dBFS  dropped=0

## Verify the Application Is Registered

Query the registry from the Client Machine:

python3 xrcomm_grpc_client.py --host <server-ip>:50051 list-ips

Each of the 8 channel slots should appear active:

slot=0  name=hello_world_ch0  pid=12345  mode=IQ_MODE  port=0  ch=0  ring=ready  active=yes
slot=1  name=hello_world_ch1  pid=12345  mode=IQ_MODE  port=0  ch=1  ring=ready  active=yes
slot=2  name=hello_world_ch2  pid=12345  mode=IQ_MODE  port=0  ch=2  ring=ready  active=yes
slot=3  name=hello_world_ch3  pid=12345  mode=IQ_MODE  port=0  ch=3  ring=ready  active=yes
slot=4  name=hello_world_ch4  pid=12345  mode=IQ_MODE  port=1  ch=4  ring=ready  active=yes
slot=5  name=hello_world_ch5  pid=12345  mode=IQ_MODE  port=1  ch=5  ring=ready  active=yes
slot=6  name=hello_world_ch6  pid=12345  mode=IQ_MODE  port=1  ch=6  ring=ready  active=yes
slot=7  name=hello_world_ch7  pid=12345  mode=IQ_MODE  port=1  ch=7  ring=ready  active=yes

# Application API Reference

Include xrcomm_ip_client.h (found in XRComm_platform_drivers/include/) in your applications. No DPDK headers are required.

## Quick API Use Case: Consume One 8T8R Channel

The typical customer application opens one logical channel, waits until the platform has created the shared-memory ring, then polls batches until the platform stops:

#include <stdio.h>
#include "xrcomm_ip_client.h"

int main(void) {
    xrcomm_ip_client_t client;
    if (xrcomm_ip_client_init_port_channel(&client, "my_ch0_app", 0, 0) != 0) {
        return 1;
    }
    if (xrcomm_ip_client_wait_ready(&client, 5000) != 0) {
        xrcomm_ip_client_shutdown(&client);
        return 1;
    }

    while (xrcomm_ip_client_running(&client)) {
        const xrcomm_iq_slot_t *slot = xrcomm_ip_client_poll(&client);
        if (!slot) {
            continue;
        }

        printf("port=%u channel=%u samples=%u\n",
               slot->hsp_port, slot->channel, slot->valid_sample_count);
        xrcomm_ip_client_report_spectrum(&client, 1, 0.0f);
        xrcomm_ip_client_consume(&client);
    }

    xrcomm_ip_client_shutdown(&client);
    return 0;
}

Run one instance per channel when you want separate per-channel processing, or follow the hello-world pattern to register all 8 channels from one process.

## Connection & Lifecycle

### xrcomm_ip_client_init_port_channel

int xrcomm_ip_client_init_port_channel(xrcomm_ip_client_t *client, 
                                       const char *name, 
                                       uint32_t hsp_port, 
                                       uint32_t channel);

8T8R: Registers the application to consume the IQ stream of a single channel of one HSP port. Arguments: client handle, application name (max 63 chars), hsp_port (0 or 1), channel (0–7). Returns 0 on success.

### xrcomm_ip_client_wait_ready

int xrcomm_ip_client_wait_ready(xrcomm_ip_client_t *client, int timeout_ms);

Blocks the caller until the platform has created and opened the shared memory data ring for this channel. Timeout of 0 means wait indefinitely. Returns 0 when ready, -1 on timeout.

### xrcomm_ip_client_running

bool xrcomm_ip_client_running(xrcomm_ip_client_t *client);

Returns true as long as the platform’s receive loop is active. Use this as the main condition for your ingest loop.

### xrcomm_ip_client_shutdown

void xrcomm_ip_client_shutdown(xrcomm_ip_client_t *client);

Deregisters the channel handle from the platform and releases all mapped shared memory regions.

## Data Consumer Functions

### xrcomm_ip_client_poll

const xrcomm_iq_slot_t* xrcomm_ip_client_poll(xrcomm_ip_client_t *client);

Polls the data ring for a new batch of IQ samples. Returns a read-only pointer to the batch if available, or NULL if empty or if IQ mode is off. Batch fields: - samples[]: Array of interleaved little-endian int16 IQ pairs - valid_sample_count: Count of valid samples in the batch - hsp_port, channel: Metadata indicating source port and channel

### xrcomm_ip_client_consume

void xrcomm_ip_client_consume(xrcomm_ip_client_t *client);

Releases the current batch slot and returns it to the platform’s write pool. Must be called once per successful poll.

## Diagnostics

### xrcomm_ip_client_get_dropped

uint64_t xrcomm_ip_client_get_dropped(xrcomm_ip_client_t *client);

Returns the number of batches dropped by the platform for this subscription because the application polled too slowly.

### xrcomm_ip_client_report_spectrum

void xrcomm_ip_client_report_spectrum(xrcomm_ip_client_t *client, 
                                      uint64_t processed, 
                                      float power_dbfs);

Reports processing stats back to the platform, making them visible in gRPC telemetry and statistics.